- Oak is priced at €875 per cubic metre. Calculate the cost of each of the three oak planks that are specified in the table below.

|  | Material | Length | Width | Thickness |
| --- | --- | --- | --- | --- |
| A) | Oak | 3.2m | 250mm | 37mm |
| B) | Oak | 2.8m | 195mm | 65mm |
| C) | Oak | 3.6m | 150mm | 50mm |

- Calculate the compound interest on €3700 for 3 years at 5.5% per annum.

- Calculate the number of risers and treads needed for a private stairs with a total rise of 2800mm and total going of 3150mm.

- The stair formula of 2R + G = 550mm to 700mm. select a suitable rise for a going of 235mm to conform to the formula.

- Eight circular tops 600mm diameter are cut from a sheet 2.440m x 1.220m. Calculate the percentage waste to the nearest whole number. (Area of circle = 3.14 x R²)

- Softwood costs €630 per m³ plus VAT at 23%. Calculate the total value of the following list allowing 7.5% for cutting and waste.

| Quantity | Length | Width | Thickness | Volume |
| --- | --- | --- | --- | --- |
| 5 | 5100mm | 225mm | 50mm |  |
| 12 | 4800mm | 175mm | 50mm |  |
| 8 | 4200mm | 100mm | 75mm |  |

- €7500 was invested at compound interest for 3 years.

The first year rate was 3.5%.

The second year rate was 3%.

The third year rate was 2.5%.

Calculate the final amount and the interest earned.

- Three circular tops 900mm diameter are cut from a sheet 2.440m x 1.220m. Calculate the percentage waste to the nearest whole number. (Area of circle = 3.14 x R²)

- Eight half round tops 850mm diameter are cut from a sheet 2.440m x 1.220m. Calculate the percentage waste to the nearest whole number. (Area of circle = 3.14 x R²)

- A contractor estimates that for every €1.50 that is spent on material, the labour costs are €4.50 and the overhead costs are €0.50. Calculate the overhead cost on a job where the total cost is €24,700.

- A brand new van cost €25,500. It depreciates by 15% in the first year and 8% in each of the next two years. What is the book value of the van after 3 years?

- Given an overall height from floor level to floor level of 3.1m, and an unrestricted going, select a suitable rise, going and pitch to comply with building regulations.

- Oak is priced at €875 per cubic metre. Calculate the cost of each of the three oak planks that are specified in the table below.

|  | Material | Length | Width | Thickness | Volume | Cost | Price |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A) | Oak | 3.2m | 250mm | 37mm | 0.0296 | 875 | € 25.90 |
| B) | Oak | 2.8m | 195mm | 65mm | 0.03549 | 875 | € 31.05 |
| C) | Oak | 3.6m | 150mm | 50mm | 0.027 | 875 | € 23.63 |

- Calculate the compound interest on €3700 for 3 years at 5.5% per annum.

€3700.00 * 1.055 = €3,903.50

€3903.50 * 1.055 = €4,118.19

€4118.19 * 1.055 = €4,344.69

- Total Compound interest = € 644.69

- Calculate the number of risers and treads needed for a private stairs with a total rise of 2800mm and total going of 3150mm.

2800/16 = 175	    2800/15 = 186.667        2800/14 = 200          2800/13=215.385         2800/12 = 233.333

(Might be possible to use 16, 15, 14 or 13 risers but must prove answer)

Example: 	16 risers @ 175 will need 15 treads @ 210  {3150/15} Not possible

15 risers @ 186.667 will need 14 treads @ 225  {3150/14}

Proof 1 ;	 2 R + G = 550 – 700		2(186.667) + 225 = 598.334

Proof 2;		Tan A = 186.667	= 0.8296	Tan⁻¹ 0.8296 = 39.68°

225

- The stair formula of 2R + G = 550mm to 700mm. select a suitable rise for a going of 235mm to conform to the formula.

2 R + G = 550 – 700	=>	2(R) = {550-235} – {700-235}

2(R) = 315 – 465

Rise = 157.5mm – 220mm (Max rise even though 465/2=232.5)

- Eight circular tops 600mm diameter are cut from a sheet 2.440m x 1.220m. Calculate the percentage waste to the nearest whole number. (Area of circle = 3.14 x R²)

Area of Sheet 	= 2.44 * 1.22		Area Required = 8 * π * 0.3²

= 2.9768				= 2.2619

- Area of waste = 0.7149m²		so	0.7149 * 100 	= 24.0157 	= 24%

2.9768

- Softwood costs €630 per m³ plus VAT at 23%. Calculate the total value of the following list allowing 7.5% for cutting and waste.

| Quantity | Length | Width | Thickness | Volume |
| --- | --- | --- | --- | --- |
| 5 | 5100mm | 225mm | 50mm | 0.286875m³ |
| 12 | 4800mm | 175mm | 50mm | 0.504m³ |
| 8 | 4200mm | 100mm | 75mm | 0.252m³ |

Total Volume = 1.042875m³

1.042875 * 1.075 (Cutting and waste) = 1.121090625m³ * €630  	= € 706.287

Plus VAT @ 23%        * 1.23

€ 868.73

- €7500 was invested at compound interest for 3 years.

The first year rate was 3.5%; The second year rate was 3%; The third year rate was 2.5%.

Calculate the final amount and the interest earned.

€7500.00 * 1.035 = €7,762.50

€7762.50 * 1.03   = €7,995.38

€7995.38 * 1.025 = €8,195.26 (Final Amount)

- Total Interest Earned = €695.26

- Three circular tops 900mm diameter are cut from a sheet 2.440m x 1.220m. Calculate the percentage waste to the nearest whole number. (Area of circle = 3.14 x R²)

Area of Sheet 	= 2.44 * 1.22		Area Required = 3 * π * 0.450²

= 2.9768				= 1.9085

- Area of waste = 1.0683m²		so	1.0683 * 100 	= 35.89	        = 36%

2.9768

- Eight half round tops 850mm diameter are cut from a sheet 2.440m x 1.220m. Calculate the percentage waste to the nearest whole number. (Area of circle = 3.14 x R²)

Area of Sheet 	= 2.44 * 1.22		Area Required = 8 * 0.5 * π * 0.425²

= 2.9768				= 2.2698

- Area of waste = 0.707m²		so	 0.707 * 100 	=23.75	       = 24%

2.9768

- A contractor estimates that for every €1.50 that is spent on material, the labour costs are €4.50 and the overhead costs are €0.50. Calculate the overhead cost on a job where the total cost is €24,700.

€1.50 : €4.50 : €0.50	=	150 : 450 : 50 	= 650 parts, 	so one part is €38  (€24,700 / 650)

- (150*€38) : (450*€38) : (50*€38)	= €5,700 : €17,100 : €1,900 (Overheads)

- A brand new van cost €25,500. It depreciates by 15% in the first year and 8% in each of the next two years. What is the book value of the van after 3 years?

€25,500 * 0.85 = €21,675.00

€21,675 * 0.92 = €19,941.00

€19,941 * 0.92 = €18,345.72 (Book Value after 3 years)

- Given an overall height from floor level to floor level of 3.1m, and an unrestricted going, select a suitable rise, going and pitch to comply with building regulations.

3100/16 = 193.75 		3100/15 = 206.667		3100/14 = 221.429

(Might be possible to use 16 or 15 risers but must prove answer)

Option 1: 	16 risers @ 193.75 will need 15 treads @ 250  (Optimum Going)

Proof 1 ;	 2 R + G = 550 – 700		2(193.75) + 250 = 637.5

Proof 2;		Tan A = 193.75	= 0.775		Tan⁻¹ 0.775 = 37.776°

250

Option 2: 	15 risers @ 206.667 will need 14 treads @ 250  (Optimum Going)

Proof 1 ;	 2 R + G = 550 – 700		2(206.667) + 250 = 663.34

Proof 2;		Tan A = 206.667 / 250	= 0.82668		Tan⁻¹ 0.82668 = 39.579°