- Oak is priced at €1,250 per cubic metre. Calculate the cost of each of the three oak planks that are specified in the table below.

|  | Material | Length | Width | Thickness |
| --- | --- | --- | --- | --- |
| A) | Oak | 4.2m | 300mm | 25mm |
| B) | Oak | 3.8m | 225mm | 50mm |
| C) | Oak | 2.6m | 150mm | 65mm |

- Calculate the compound interest on €4,200 for 3 years at 3.75% per annum.

- Calculate the number of risers and treads needed for a private stairs with a total rise of 2950mm and total going of 3200mm.

- The stair formula of 2R + G = 550mm to 700mm. select a suitable rise for a going of 360mm to conform to the formula.

- Twelve circular tops 400mm diameter are cut from a sheet 2.440m x 1.220m. Calculate the percentage waste to the nearest whole number. (Area of circle = 3.14 x R²)

- Softwood costs €540 per m³ plus VAT at 23%. Calculate the total value of the following list allowing 7.5% for cutting and waste.

| Quantity | Length | Width | Thickness | Volume |
| --- | --- | --- | --- | --- |
| 10 | 4800mm | 150mm | 50mm |  |
| 6 | 4800mm | 150mm | 75mm |  |
| 14 | 4200mm | 75mm | 38mm |  |

- €11,000 was invested at compound interest for 3 years.

The first year rate was 5.5%.

The second year rate was 4.5%.

The third year rate was 4%.

Calculate the final amount and the interest earned.

- Five semi-circular tops 780mm diameter are cut from a sheet 2.440m x 1.220m. Calculate the percentage waste to the nearest whole number. (Area of circle = 3.14 x R²)

- Six half round tops 1200mm diameter are cut from a sheet 2.440m x 1.220m. Calculate the percentage waste to the nearest whole number. (Area of circle = 3.14 x R²)

- A contractor estimates that for every €1.00 that is spent on material, the labour costs are €4.00 and the overhead costs are €0.30. Calculate the overhead cost on a job where the total cost is €26,500.

- A brand new van cost €32,000. It depreciates by 12.5% in the first year and 7.5% in each of the next two years. What is the book value of the van after 3 years?

- Given an overall height from floor level to floor level of 3.250m, and an unrestricted going, select a suitable rise, going and pitch to comply with building regulations.

- Oak is priced at €1,250 per cubic metre. Calculate the cost of each of the three oak planks that are specified in the table below.

|  | Material | Length | Width | Thickness | Volume | Cost | Price |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A) | Oak | 4.2m | 300mm | 25mm | 0.0315 | 1250 | € 39.38 |
| B) | Oak | 3.8m | 225mm | 50mm | 0.04275 | 1250 | € 53.44 |
| C) | Oak | 2.6m | 150mm | 65mm | 0.02535 | 1250 | € 31.69 |

- Calculate the compound interest on €4200 for 3 years at 3.75% per annum.

€4200.00 * 1.0375 = €4,357.50

€4357.50 * 1.0375 = €4,520.91

€4520.91 * 1.0375 = €4,690.44

- Total Compound interest = € 490.44

- Calculate the number of risers and treads needed for a private stairs with a total rise of 2950mm and total going of 3200mm.

2950/16 = 184.375	2950/15 = 196.667	2950/14 = 210.714	 2950/13 = 226.923

(Possible to use 16, 15 or 14 risers but must prove answer)

Example: 	14 risers @ 210.714 will need 13 treads @ 246.15  {3200/13}

Proof 1 ;	 2 R + G = 550 – 700		2(210.714) + 246.15 = 667.578

Proof 2;		Tan A = 210.714	= 0.856		Tan⁻¹ 0.856  = 40.565°

246.15

- The stair formula of 2R + G = 550mm to 700mm. select a suitable rise for a going of 360mm to conform to the formula.

2 R + G = 550 – 700	=>	2(R) = {550-360} – {700-360}

2(R) = 190 – 340

Rise = 95mm – 170mm (optimum of 175mm wouldn’t work)

- Twelve circular tops 400mm diameter are cut from a sheet 2.440m x 1.220m. Calculate the percentage waste to the nearest whole number. (Area of circle = 3.14 x R²)

Area of Sheet 	= 2.44 * 1.22		Area Required = 12 * π * 0.200²

= 2.9768				= 1.50796

- Area of waste = 1.46884m²		so	1.46884 * 100 	= 49.31	     = 49%

2.9768

- Softwood costs €540 per m³ plus VAT at 23%. Calculate the total value of the following list allowing 7.5% for cutting and waste.

| Quantity | Length | Width | Thickness | Volume |
| --- | --- | --- | --- | --- |
| 10 | 4800mm | 150mm | 50mm | 0.360m³ |
| 6 | 4800mm | 150mm | 75mm | 0.324m³ |
| 14 | 4200mm | 75mm | 38mm | 0.16758m³ |

Total Volume = 0.85158m³

0.85158 * 1.075 (Cutting and waste) = 0.9154485m³ * €540	= € 494.34

Plus VAT @ 23%       * 1.23

€ 608.04

- €11,000 was invested at compound interest for 3 years.

The first year rate was 5.5%; The second year rate was 4.5%; The third year rate was 4%.

Calculate the final amount and the interest earned.

€11000.00 * 1.055 = €11,605

€11605.00 * 1.045 = €12,127.23

€12127.23 * 1.040 = €12,612.31 (Final Amount)

- Total Interest Earned = €1,612.31

- Five semi-circular tops 780mm diameter are cut from a sheet 2.440m x 1.220m. Calculate the percentage waste to the nearest whole number. (Area of circle = 3.14 x R²)

Area of Sheet 	= 2.44 * 1.22		Area Required = 5 * 0.5 * π * 0.390²

= 2.9768				= 1.1946

- Area of waste = 1.7822m		so	1.7822 * 100 	= 59.87	     = 60%

2.9768

- Four half round tops 1200mm diameter are cut from a sheet 2.440m x 1.220m. Calculate the percentage waste to the nearest whole number. (Area of circle = 3.14 x R²)

Area of Sheet 	= 2.44 * 1.22		Area Required = 4 * 0.5 * π * 0.600²

= 2.9768				= 2.26195

- Area of waste = 0.71485m²		so	0.71485 * 100 	= 24.01	      = 24%

2.9768

- A contractor estimates that for every €1.00 that is spent on material, the labour costs are €4.00 and the overhead costs are €0.30. Calculate the overhead cost on a job where the total cost is €15,300.

€1.00 : € 4.00 : €0.30	=	100 : 400 : 30 	= 530 parts, 	so one part is €50  (€26,500 / 530)

- (100*€50) : (400*€50) : (30*€50)	= €5,000 : €20,000 : €1,500 (Overheads)

- A brand new van cost €32,000. It depreciates by 12.5% in the first year and 7.5% in each of the next two years. What is the book value of the van after 3 years?

€32,000 * 0.875 = €28,000.00

€28,000 * 0.925 = €25,900.00

€25,900 * 0.925 = €23,957.50 (Book Value after 3 years)

- Given an overall height from floor level to floor level of 3.250m, and an unrestricted going, select a suitable rise, going and pitch to comply with building regulations.

3250/16 = 203.125 		3250/15 = 216.667		3250/14 = 232.143

(Might be possible to use 16 or 15 risers but must prove answer)

Option 1: 	16 risers @ 203.125 will need 15 treads @ 250  (Optimum Going)

Proof 1 ;	 2 R + G = 550 – 700		2(203.125) + 250 = 656.25

Proof 2;		Tan A = 203.125        = 0.8125		Tan⁻¹ 0.8125 = 39.094°

250

Option 2: 	15 risers @ 216.667 will need 14 treads @ 250  (Optimum Going)

Proof 1 ;	 2 R + G = 550 – 700		2(216.667) + 250 = 683.34

Proof 2;		Tan A = 216.667        = 0.8667		Tan⁻¹ 0.8667 = 40.914°

250